# OCPP 2.0.1 Compatibility Evaluation

EVSecSim Testbed - PGTwin Grid Integration

Date: 2026-06-07   |   Repository: SUTDEVsec (branch: main)   |   Author: Bryan Wang

# Question

Can the current grid accept OCPP 2.0.1 input?

# Summary

The answer splits into three tiers - and the most important one dissolves the question. The grid accepts OCPP 2.0.1 input today with zero changes because it only ever sees a kW float, never an OCPP message. The CSMS does not accept 2.0.1, being version-locked to OCPP 1.6 at both the handler and schema level (proven empirically below). Bridging the gap is a moderate, dependency-free addition, since the ocpp.v201 module is already installed.

# Tier 1 - The grid itself: trivially yes (it never sees OCPP)

pgtwin (ZSGplussync_docker.py) does not parse OCPP at all. It reads a single kW float from /shared/ev_load_kw.txt before each runpp() call. That file is written by the attack process, not by the CSMS:

attack script --(speaks OCPP x.y)--> CSMS
      |
      +--(writes plain float)--> /shared/ev_load_kw.txt --> pgtwin

The grid is protocol-agnostic by construction. It accepts a number. Whether that number originated from OCPP 1.6, 2.0.1, MQTT, or a shell echo is invisible to it. No grid change is needed for 2.0.1.

# Tier 2 - The current CSMS, as-is: no

This is the precise claim, and it was demonstrated rather than asserted. The live CSMS was run and sent real 2.0.1 frames over the wire. The results:

| Frame sent | CSMS wire response |
| --- | --- |
| v2.0.1 TransactionEvent | CALLERROR - NotSupported |
| v2.0.1 BootNotification (chargingStation/reason payload) | CALLERROR - FormatViolation |
| v1.6 BootNotification (control) | CALLRESULT - Accepted |

Two independent reasons it rejects 2.0.1:

Handler gap - csms_server4.py is built on "from ocpp.v16 import ChargePoint" with @on(StartTransaction), @on(MeterValues), etc. OCPP 2.0.1 replaces all three with a unified TransactionEvent, which hits no handler and returns NotSupported.

Schema gap - even BootNotification, whose name exists in both versions, carries a different payload in 2.0.1 (chargingStation/reason vs the flat chargePointVendor/chargePointModel) and fails v16 schema validation before the handler runs, returning FormatViolation.

# Tier 3 - A v201 (or dual-stack) CSMS + v201 clients: feasible, no new dependency

The installed library already ships the 2.0.1 implementation:

ocpp==2.1.0  ->  submodules: ['experimental', 'v16', 'v201', 'v21']
from ocpp.v201 import ChargePoint, call, call_result   # imports clean

Enabling 2.0.1 is therefore application code, not a dependency change. It requires a v201 CSMS (or a version-dispatching one) plus v201 attack/EVSE scripts. The grid-coupling line stays identical.

# Two caveats worth knowing

The handshake is non-standard and version-independent. Both the CSMS and clients pass the charge-point ID as the first raw WebSocket frame (await ws.send("CP_1")) with no Sec-WebSocket-Protocol negotiation. Real OCPP-J negotiates ocpp1.6 vs ocpp2.0.1 and puts the CP ID in the URL path. The current framing is a shortcut - it neither blocks nor helps 2.0.1, but it is not "real" OCPP framing for either version.

"Accept input" is not "mitigate attacks." If the interest is whether 2.0.1 stops the grid attacks: 2.0.1 would kill Attack 5 (signed firmware) and Attack 9 (signed StopTransaction), but Attack 7/8 still work - a compromised-but-authenticated charger can still report inflated MeterValues, and 2.0.1 has no physical-plausibility check on reported power either. That is a separate study.

# Bottom line

The grid accepts 2.0.1 input today with zero changes (it only sees a float). The CSMS does not - it is v16-locked at both the handler and schema level, proven by the NotSupported / FormatViolation responses above. Bridging the gap is a moderate, dependency-free addition since ocpp.v201 is already installed.

# Appendix - Empirical probe method

A raw WebSocket client connected to the running CSMS and sent hand-crafted CALL frames, capturing the exact wire response (avoiding any client-side library schema validation so the server behaviour is shown directly):

CALL = 2;  CALLERROR = 4
ws.send('PROBE-CP-201')          # non-standard first-frame handshake

# 2.0.1 TransactionEvent (unknown action to v16 CSMS)
[2,'p1','TransactionEvent', { ... eventType:'Started' ... }]
  -> [4,'p1','NotSupported', ...]

# 2.0.1 BootNotification (name shared, payload differs)
[2,'p2','BootNotification', {'chargingStation':{...}, 'reason':'PowerUp'}]
  -> [4,'p2','FormatViolation', ...]

# control: valid 1.6 BootNotification
[2,'p3','BootNotification', {'chargePointVendor':'Vendor', 'chargePointModel':'Model1'}]
  -> [3,'p3', {'status':'Accepted', 'interval':10, ...}]